## World Record Attempt

Class F (Aeromodels)

Sub-class F5-S (Aeroplane, electric motor, rechargeable sources of current)

Category: F5: Radio Controlled Flight

No 173: Gain in Altitude

Record Claim: 2,573 m (8441 ft), date 9/11/2003 - yet to be ratified.

# “Big Bird”

# Photo courtesy Julie Bird

- Airframe: Construction is conventional - balsa, ply where needed, tissue covering, fibreglass hatch/cowl.

- Span: 328.8cm

- Root cord: 39.2cm

- Tip cord: 29cm

- Surface Areas: Wing 119dm sq, Tail 18dm sq

- Weight: 4200gms to 4940gms

- Motor: Actro 40/6 Brushless, 18.75" x 11" Prop

- Motor Battery: 9 series x 4 parallel eTech 1200 Lithium Polymer cells or 28 x GP3300 NiMh cells

- Radio Control system: JR 3810 TX, JR649PCM Rx, 6 x JR 381 servos
Controls: Elev, Rud, Ail, Flaps, Motor ESC (Co-Pilot infrared sensor for pitch stability)

- Climb Rate: Full throttle = 1300 ft per min approx., 70% Throttle = 700 ft per min (70% throttle used for record attempt) 
Other Equipment:

- Head mounted binoculars, Bushnell 5 x 25 extra wide view

- Garmin eTrex GPS with barometric altimeter

- Also EMS Altitude Data logger as second barometric altimeter

- Data Capture card and Telemetry Module

Total flight time 31 minutes. (12 min ascent, 19 min descent)

Current World record is: 2'200 m (7218 ft)

Date of flight: 19/11/2000
Pilot: Wolfgang SCHAEPER (Germany)
Course/place: Markdorf (Germany)